# DF 403

## New Product Release

Automatic Re-ignitor Kit for 80/40H and 40/40H Vaporizers       - 9 VDC -

(ASDI P/N: 41011)

Benefits:

- Eliminates vaporizer shut-downs due to pilot outages during windstorms or blow-outs

- Allows operator to start vaporizer without opening cabinet door

Description:

Self contained electric spark re-ignition kit for use on all Algas and Algas-SDI 40/40H and 80/40H vaporizers. Requires 7/8” drill or ½” conduit punch for mounting. Mounts to side of cabinet.

Comes with:

- NEMA 12 enclosure for easy field mounting

- Start switch for easy lighting without removing vaporizer door

- Electrode assembly that mounts directly to existing pilot

- 9VDC battery

List Price:  $234 USD

Operating Principle:  Simply depress the operating switch to engage the ignitor. The electrode then creates a rapid spark in the pilot gas zone to light the pilot burner. As the pilot flame is established, a small current is conducted through the flame and back to ground, which in turn suppresses the sparking mechanism. Should the pilot flame extinguish, the spark will resume to re-establish the pilot flame. Batteries typically last 6-12 months depending on use.