...Innovative liquid vaporizing and gas mixing solutions

Bulletin Number:  98/03  Rev. 1

ALL PERSONS INSTALLING ALGAS REPAIR KIT NO. 3-0004

ALL EQUIPMENT USING EQUIMETER (ROCKWELL) MODEL 041C AND 046C

In order to reduce the number of repair kits and to simplify repairs performed by field service technicians, Algas-SDI has developed a repair kit suitable for both the Equimeter (previously Rockwell) Model 041C and the 046C Regulators.  The Model 046C regulator is the replacement for the Model 041C regulator, which is now obsolete.

As of June 20, 1998, when servicing the regulator (1) identify which model you have by the nameplate mounted on the spring housing.  If you have an 041C regulator be sure to use the diaphragm marked with Algas part number 8147-101 (Equimeter part number 141-62-150-01).  If you have an 046C regulator be sure to use the diaphragm marked with Algas part number 3-3148 (Equimeter part number 046-00-150-00).  The remaining three parts fit both regulators.

ALGAS-SDI REPAIR KIT 3-0004 ON ALGAS-SDI MIXER REGULATORS

1 ea. P/N 8147-101		Diaphragm, Buna N, Equimeter #141-62-150-01 (for 041C regulator)

1 ea. P/N 3-3148		Diaphragm, Buna N, Equimeter #046-00-150-00 (for 046C regulator)

1 ea. P/N 3-3149		Valve disk, Vitron, Equimeter #141-62-511-04 (for both regulators)

1 ea. P/N 504-01		Tetraseal (O-ring), Buna N, Equimeter #902497 (for both regulators)

1 ea. P/N 3-3125		Boost tube, Equimeter #141-62-045-00 (for both regulators)

1 ea.		Service Bulletin No. 98.03 Rev. 1

At Algas-SDI International we are committed to continuous improvement and customer satisfaction.  We hope you will find the results of this bulletin useful and beneficial.  If you have any problems or questions please feel free to call at 206-789-5410.

1140 NW 46 Street, Seattle, Washington, USA 98107

Tel:  206-789-5410	Fax: 206-789-5414	Web: www.algas-sdi.com

Revision:  December 15, 1999	FILE: Regulator Service Bulletin Bulletin.doc

B/N:   98-03  Rev. 1